Action full title:

Universal, mobile-centric and opportunistic communications architecture

Action acronym:

UMOBILE

Deliverable:

D3.3 “UMOBILE ICN layer abstraction initial specification”

Project Information:

| Project Full Title | Universal, mobile-centric and opportunistic communications architecture |
| --- | --- |
| Project Acronym | UMOBILE |
| Grant agreement number | 645124 |
| Call identifier | H2020-ICT-2014-1 |
| Topic | ICT-05-2014 Smart Networks and novel Internet Architectures |
| Programme | EU Framework Programme for Research and Innovation HORIZON 2020 |
| Project Coordinator | Prof. Vassilis Tsaoussidis, Democritus University of Thrace |

Deliverable Information:

| Deliverable Number-Title | D3.3: UMOBILE architecture: high-level initial specification |
| --- | --- |
| WP Number | WP3 |
| WP Leader | DUTH |
| Task Leader (s) | UCL |
| Authors | COPELABS: Paulo Mendes, Waldir Moreira DUTH: Nikolaos Bezirgiannidis, Sotiris Diamantopoulos, Ioannis Komnios, Vassilis Tsaoussidis UCL: Ioannis Psaras, Sergi Rene UCAM: Jon Crowcroft, Adisorn Lertsinsrubtavee, Arjuna Sathiaseelan TECNALIA: Susana Perez Sanchez, Iñigo Sedano SENCEPTION: Rute Sofia |
| Contact | i.psaras@ucl.ac.uk, s.rene@ucl.ac.uk |
| Due date | M12: 31/01/2016 |
| Actual date of submission | --/--/2016 |

Dissemination Level:

| PU | Public | x |
| --- | --- | --- |
| CO | Confidential, only for members of the consortium (including the Commission Services) |  |
| CI | Classified, as referred to in Commission Decision 2001/844/EC |  |

Document History:

| Version | Date | Description |
| --- | --- | --- |
| Version 0.1 | 11/01/16 | First draft to the consortium for alignment with D3.3 and allocation of contributions |
| Version 0.2 | 22/01/16 | Second draft to the consortium for alignment with D3.3 and allocation of contributions |
| Version 0.3 | 1/2/16 | Third draft to the consortium for alignment with D3.3 and allocation of contributions |
| Version 1.0 | 3/2/16 | Final version |
| Version 1.1 | 4/11/16 | First draft to the consortium of the D3.3 amendment after the midterm review |
| Version 1.2 | 10/11/16 | Second draft to the consortium of the D3.3 amendment after the midterm review |

Table of Contents

List of Definitions	6

Executive Summary	11

1	Introduction	13

2 UMOBILE architecture: Application/Service perspective	20

2.1.	UMOBILE applicability	20

2.1.1.	Micro-blogging	20

2.1.2.	Emergency situation	20

2.1.3.	Civil protection	20

2.1.4.	Social routine improvement	21

2.2.	Network services	21

2.2.1.	Network scenarios	23

2.2.2.	Communication models	25

2.2.3.	Context services	27

2.2.4.	Data services	27

3	UMOBILE architecture design	29

3.1	UMOBILE devices	30

3.2	UMOBILE services	35

3.3	Network requirements	38

4	UMOBILE modules	40

4.1	NDN plane	40

4.1.1.	Naming	40

4.2.	Context manager	43

4.2.1.	Usage context	43

4.2.2.	User context	44

4.3.	Connectivity manager	45

4.4.	UMOBILE forwarding strategies	46

4.5.	Social routing	50

4.6.	INRPP - Flow control	52

4.7.	Service pre-fetching	52

4.7.1.	Service manager	52

4.7.2.	Monitoring	53

4.7.3.	Service repository	53

5	UMOBILE platform WP planning	54

6	UMOBILE network requirements	56

7	Conclusion	59

References	60

List of Definitions

| Term | Meaning |
| --- | --- |
| DTN | Delay Tolerant Network (DTN) is an emerging technology that supports interoperability of other networks by accommodating long disruptions and delays between and within those networks. DTN operates in a store-and-forward fashion where intermediate nodes can temporary keep the messages and opportunistically forward them to the next hop. This inherently deals with temporary disruptions and allows connecting nodes that would be disconnected in space at any point in time by exploiting time-space paths. |
| ICN | Information-Centric Network (ICN) has emerged as a promising solution for the future Internet's architecture that aims to provide better support for efficient information delivery. ICN approach uniquely identifies information by name at the network layer, deploys in-network caching architecture (store information at the network node) and supports multicast mechanisms. These key mechanisms facilitate the efficient and timely information (contents and services) delivery to the end-users. |
| Content | Content refers to a piece of digital information that is disseminated and consumed by end-user equipment. |
| Node | A wireless or wired capable device. |
| User | An entity (individual or collective) that is both a consumer and a relay of user services. |
| User Service | Context-aware services are considered as a set of mechanisms that assist incorporating information about the current surrounding of mobile users in order to provide more relevant of services. |
| User Interest | A parameter capable of providing a measure (cost) of the “attention” of a user towards a specific (piece of) information in a specific time instant. Particularly, users can cooperate and share their personal and individual interests that enable the social interactions and data sharing across multiple users. |
| User Requirement | User requirement corresponds to the specifications that users expect from the application. |
| Upstream | Upstream traffic refers to outgoing data such as short message, photo or uploading video clips that are sent from user equipment. |
| Downstream | Downstream traffic refers to data is obtained by use equipment from network. This includes downloading files, web page, receiving messages, etc. |
| Gateway | Gateway typically means an equipment installed at the edge of a network. It connects the local network to larger network or Internet. In addition, gateway also has a capability to store services and contents in its cache to subsequently provide local access communication. |
| Customer Premises | Customer Premises relate to residential households and enterprise market and are, as of today, controlled by the end-user. |
| User-centric | User-centric refers to a new paradigm leveraging user information at large to deliver novel content or services by users towards other users. |
| UMOBILE System | UMOBILE System refers to an open system that provides communication access to users through wired or wireless connectivity. This system exploits the benefit of local communication to minimize upstream and downstream traffic. The services or contents can be exchanged and stored in several devices such as gateways; user equipments; customer premises equipments such as WiFi Access Points in order to efficiently delivery the desired contents or services to end-users. |
| UMOBILE Architecture | A mobile-centric service-oriented architecture that efficiently delivers contents and services to the end-users. The UMOBILE architecture integrates the principles of Delay Tolerant Networks (DTN) and Information-Centric Networks (ICN). |
| User-equipment | User-equipment (UE) corresponds to a generic user terminal (for example smart phone or notebook). In terms of UE and for operating systems we consider mainly smartphones equipped with Android; notebooks with UNIX, Windows, Mac OS. |
| Social Trust | Trust which builds upon associations of nodes is based on the notion of shared interests; individual or collective expression of interests; affinities between end-users. |
| Application | Computer software design to perform a single or several specific tasks, e.g. a calendar and map services. In UMOBILE, context-aware applications are considered. |
| Incentive | A factor (e.g., economic or sociological) that motivates a particular action or a preference for a specific choice. |
| Service | Service refers to a computational operation or application running on the network which can fulfil an end-user’s request. The services can be hosted and computed in some specific nodes such servers or gateways. Specifically, service is normally provided for remuneration, at a distance, by electronic means and at the individual request of a recipient of services. For the purposes of this definition;“at a distance” means that the service is provided without the parties being simultaneously present; “by electronic means” means that the service is sent initially and received at its destination by means of electronic equipment for the processing (including digital compression) and storage of data, and entirely transmitted, conveyed and received by wire, by radio, by optical means or by other electromagnetic means; “at the individual request of a recipient of services” means that the service is provided through the transmission of data on individual request. Refer to D2.2 for further details. |
| Trust Association | An unidirectional social trust association between two different nodes. |
| UMOBILE gateway | Role (software functionality) which reflects an operational behavior making a UMOBILE device capable of acting as a mediator between UMOBILE systems and non-UMOBILE systems – the outside world. |
| UAV | Unmanned Aerial Vehicle, which is an aircraft with no pilot on board. |
| BSS | Basic Service Set is a set consisting of all the devices associated with a consumer or enterprise IEEE 802.11 wireless local area network (WLAN). The service set can be local, independent, extended or mesh. Service sets have an associated identifier, the Service Set Identifier (SSID), which consists of 32 octets that frequently contains a human readable identifier of the network. |
| DC | Data Cache is responsible for holding information concerning the content carried by the current node. |
| CIT | Carried Interest Table is responsible for keeping up-to-date information concerning the data interests of the current node along with its social weights towards other nodes with whom it socially interacts. |
| SWM | Social Weight Measurer is responsible for keeping track of the contact duration between the current and encountered nodes. |
| SWR | Social Weight Repository is responsible for storing the list of interests the current node comes across (obtained upon encountering a peer). |
| tPIT | temporary Pending Interest Table holds the list of interests seen by the encountered node and the social weights between itself and such interests. |
| tCDT | Temporary Carried Data Table holds information about the data that the encountered node is currently carrying. |
| SWCDG | Social Weight and Carried Data Gatherer is responsible for obtaining the list of interests and social weights towards the encountered node. |
| DM | Decision Maker is responsible for deciding whether replication should occur based on the level of social interaction towards specific interests, based on the SCORP algorithm. |
| SC | Service Controller manages the mapping of publishers of services and subscribers of services in the service migration module. |
| SP | Service Publisher refers to the original content producer in the service migration module. |
| SEG | Service Execution Gateway is the point of attachment for clients in the service migration module. |
| FN | Forwarding Node is responsible for routing requests for services towards the available copies in the service migration module. |
| NDN | Named Data Networking |

Executive Summary

This document covers the UMOBILE architecture ICN layer specification, as well as UMOBILE services that apply to scenarios detailed in D2.1, for the specification of the UMOBILE architecture. This document incorporates the first version of the UMOBILE architecture high-level specification and the final document D3.4 (to be provided on M30) will describe the refined specification.

The final goal of this document is to provide a high-level description of the overall specification of the UMOBILE architecture. Special attention is given to the needed alignment with the Delay-Tolerant Network (DTN) architecture [1], and the most relevant proposal for an information-centric network architecture (ICN) [2] that fits the identified UMOBILE system and network requirements. The provided specification drives the development and implementation of the UMOBILE architecture, to be fully described in deliverable D3.1 (e.g. core platform, API, protocols, software).

The starting point for this report are deliverables D2.1 [3] and D2.2 (to be submitted on M14), which describes typical accessibility scenarios in different environments, namely, urban, remote and disaster areas, as well as requirements from the end-user (D2.1) and the system (D2.2) perspective. The UMOBILE architecture specification, provided in this document, will be complemented by the following deliverables:

- D3.1/D3.2 with a detailed specification of the UMOBILE architecture and the implementation of it.

- D3.4 with a final version of the high-level specification described in this document.

- D4.1 and D4.2 with a detailed specification of the proposed rate-regulation scheme.

- D4.3 with a detailed specification of the proposed mobile name-based replication scheme aimed at improving certain services by using priorities.

- D4.4 with a detailed specification of the different supported services and the QoS mechanisms in the UMOBILE project.

- D4.5 with a detailed specification about sensing data collection and users’ behaviour inference.

The methodology used in this deliverable is as follows:

- Start from the applicability pictures identified in D2.1 and derive application services that must be implemented using the UMOBILE architecture.

- Identify the network services required for the services previously detailed.

- Describe the high-level design of UMOBILE architecture based on the overall assumptions and requirements, as well as the capabilities provided by the selected DTN and ICN architectures.

- Detail the different modules and describe the full specification of the UMOBILE ICN layer abstraction architecture, highlighting the components that need to be further described in D3.1, D4.1/D4.2, D4.3, D4.4 and D4.5

# Introduction

The main objective of UMOBILE is to develop a mobile-centric service oriented architecture that efficiently delivers content/service to the end-users. UMOBILE decouples services from their origin locations, shifting the host-centric paradigm to a new paradigm, one that incorporates aspects from both information-centric and opportunistic networking with the ultimate purpose of delivering an architecture focused in: i) improving aspects of the existing infrastructure; ii) improving the social routine of Internet users via technology-mediated approaches; iii) extending the reach of services to areas with little or no infrastructure (e.g., remote areas, emergency situations).

UMOBILE also aims to push network services (e.g., mobility management, intermittent connectivity support) and user services (e.g., pervasive content management) as close as possible to the end-users. By pushing such services closer to the users, we can optimize, in a scalable way, aspects such as bandwidth utilization and resource management. We can also improve the service availability in challenged network environments. For example, users in some areas may suffer from intermittent and unstable Internet connectivity when they are trying to access the services.

To achieve this, the proposed UMOBILE architecture combines two emerging architecture and connectivity approaches: Information Centric Networking (ICN) and Delay Tolerant Networking (DTN). The aim is to build a novel architecture that defines a new service abstraction that brings together both information centric as well as delay tolerant networking principles into one single abstraction. We further integrate social trust computation and cooperative incentive modelling into the architecture that will enable priority dissemination of information based on the notion of smart trust circles in opportunistic communication environments. Such an abstraction would enable network services to pervasively operate in any networking environment, independently of the underlying communication technology. Such abstraction also allows innovative application and services development, providing access to data independently of the level of end-to-end connectivity available.

In the scope of WP3 – System and architecture development- we previously defined a set of objectives aimed at specifying a full node architecture. These goals are:

- Design adequate delay-tolerant interfacing for underlying protocols that efficiently utilises the available resources for a challenged and opportunistic network environment.

- Provide service abstraction to applications by incorporating the notion of information-centric networking and named-data contents.

- Establish an overall network and system architecture.

- Implement an integrated prototype platform that can be used for the various deployment alternatives.

In this document we define the high-level design of this delay-tolerant and opportunistic data-centric architecture that will provide the service abstraction to applications by using ICN principles. However, we point out that the last point of the objectives about implementation and integration will be detailed in further deliverables (D3.1 and D3.2) of WP3 and WP5 (Integration and Validation).

Most DTNs (i.e. RFC 5050 compliant [4]) rely on a host centric routing mechanism, passing bundles between nodes regardless of the data being exchanged. In contrast, ICNs allow the network to gain a better understanding of the data itself, enabling it to be easily cached and reused. Such support offers a huge potential in disrupted environments that rarely allow two hosts to reach each other.  In [5], the authors identify similarities between ICN and DTN that seem to indicate that the integration of the two architectures is a logical progression, with the potential to combine the benefits of both. Importantly, we believe that the prominence of disconnectivity in daily situations (e.g. underground transport, poorly provisioned areas, during expensive roaming) means that any future Internet architectures must place a high priority on supporting delay tolerance.   Clearly, these examples motivate the need for an integrated architecture in opportunistic mobile networks.

In order to provide a new UMOBILE information-centric network architecture with delay-tolerant and opportunistic communications requirements, we will use the Named-Data Networking (NDN) [8] architecture as an initial approach. NDN will provide all the ICN abstractions required and we will extend it in order to provide new opportunistic, delay tolerant and social-aware features. We have chosen NDN instead of other ICN architectures (PURSUIT, DONA, Netinf [2]) because NDN has been wider accepted by the research community as a de facto standard for ICN. In the documents and Internet drafts of the IRTF Information-Centric Networking Research Group (ICNRG) [23] we can see that most of the community research efforts are focused on the NDN architecture.

NDN is an information-centric architecture that decouples content from network location. This decoupling is done by providing a new content hierarchical naming and content routing is based on these content identifiers, instead of location identifiers such as in IP. Routing is performed using similar algorithms to current IP infrastructure, utilising longest prefix matching with hierarchical aggregation to ensure scalability, using a similar Forwarding Information Base (FIB) than IP. In NDN, a content request is issued by sending an Interest packet, which is routed through the network to the closest instance of the content. Subsequently, if available, the source responds with a data packet, which follows the reverse path back to the requester using “breadcrumbs” left in a Pending Interest Table (PIT) on each router. NDN also takes benefit of a Content Store (CS) where the data packets are cached in order to be resent when other users request the same packet that has been previously cached. This way, NDN can be more efficient allowing in-network routers to provide the content instead of the end-point source, and providing inherent support to multicast communications.

Figure 1: NDN Architecture [8]

Using an NDN-based architecture provides a set of features beneficial for mobile and wireless networks that are the following:

- Native host multihoming: NDN does not bind a particular flow with a network interface. This provide a higher flexibility to use all the interfaces available to retrieve the data requested at the same time.

- Detachment of applications from location-oriented addresses: In NDN there is no necessity to force applications to take on location-oriented information. Instead, it detaches the application from such concerns. This allows the application to abstractly publish or consume content, without the need to store (or even know) its own network-layer address.

- No connection-oriented sessions: In NDN there are no end-to-end sessions and all the communications are constrained at the network level. This removal of connection-oriented protocols allows the relocation of a host without the necessity of re-establishing a connection.

- Resilience through replication: Allowing local copies of the content to be retrieved, improves network performances and mitigates the effect of failures in mobile networks.

Despite the advantages of NDN in front of the IP architecture in terms of mobility, there is still some research challenges that need to be addressed in terms of mobility [24]. These research challenges are mainly the provider mobility, pairwise path routing (breadcrumb approach), request staleness or security and privacy. Moreover, there is no extensive research in the field of opportunistic wireless networks.  Previous work on wireless networks has been focused on MANET [25], where machines are not constrained devices, or has been focused on wireless sensor networks and sink-centric data traffic [26] (i.e., sensor-to-sink or sink-to-sensor). In this project we will go one step further and propose a universal, mobile-centric and opportunistic communications architecture that will exploit any communication opportunity between mobile devices to forward data towards receivers.

In the UMOBILE project we propose a new data networking architecture that will integrate an ICN and a DTN approach. In order to provide delay-tolerant and opportunistic capabilities to the NDN architecture, in UMOBILE we will provide to NDN new forwarding mechanisms able to perform in opportunistic networks by using new logic faces, such as DTN faces, but also taking benefit of Device-to-Devices (D2D) communications, enabling application sharing between users and opportunistic routing that will use social information to reach users without the participation of the network infrastructure.

UMOBILE needs to support various challenged scenarios such as aftermath of disasters or in rural/remote network deployments or networks with limited backhaul capacity. Such challenged environments pose several challenges such as increased latency, intermittent connectivity etc. To address these challenges, we also propose a resilient service migration module which utilizes advances in lightweight operating systems to push service instances right to the network edge, similar to the new fog edge computing paradigm [27]. Instead of hosting the services in static location (fixed server), the service is considered as mobile content which can be accessed and executed anywhere within a local network. Inside a local network it utilizes a name based routing/forwarding strategy, which brings in several benefits of ICN. Within the UMOBILE project, we utilise the NDN abstractions to carry out the name based routing/forwarding strategies. We also envision that our service migration platform also makes benefit of the underlying DTN to provide resilient access to services during periods of intermittent connectivity.  In this challenged scenario, we also propose a new forwarding mechanism aimed at prioritising emergency information over less critical data, restricted to certain areas and time-space, in order to spread emergency information when no or partial infrastructure is available.

Another big issue in ICN is scalability due to name-based routing. Moving the address space from IP addresses to content names brings scalability issues to a whole new level, due to two reasons. First, name aggregation is not as trivial a task as the IP address aggregation in BGP routing. Second, the number of addressable contents in the Internet is several orders of magnitude higher than the number of IP addresses. For this reason, we also present an ICN keyword-based naming scheme aimed to help disseminating local cached information or available services in UMOBILE scenarios.

At last but not least, UMOBILE will provide to users new contextualisation services…

The UMOBILE architecture defines an ICN architecture extending NDN architecture [8] with a context detection (different kind of services and/or mobile scenarios), enabling opportunistic and social routing, opportunistic application sharing and service pre-fetching at the end of the network. Since the network requirements of the list of UMOBILE-enabled devices are different (network vs end-user devices). We present two versions of the UMOBILE architecture: one for the mobile end-users and another for the rest of devices. In Figure 2 we can observe the different components that the UMOBILE architecture will provide to extend NDN to the mobile end-users. The functionalities of these components are respectively detailed in the Section 4. We can observe that network abstractions to the applications will be: application sharing, UMOBILE forwarding (including opportunistic forwarding and emergency channel) and push services. Note that the figures are conceptual diagrams of the UMOBILE architecture and the complete relational diagram providing all the relationships between modules with a full detail will be provided in D3.1.

Figure 2 – UMOBILE architecture for mobile end-users

In Figure 3 we can observe the different components that the UMOBILE architecture will provide to extend NDN to the network devices such as routers, gateways, surrogates devices (e.g. UAVs) and hotspots (APs). The functionalities of these components are respectively also detailed in the Section 4. The main difference is that this version does not include the components aimed at contextualisation and Device-to-Device (D2D) communications. On the other side, this version includes the Service pre-fetching modules and the INRPP protocol. We can observe that network abstractions to the applications will be: Service migration, UMOBILE forwarding and Push Services.

Figure 3 – UMOBILE architecture for network devices: routers, gateways, surrogates and hotspots

The set of different UMOBILE services aimed at extending NDN to provide the northbound services are initially described in Section 2.2.5 and will be detailed in deliverable D3.1.

The document is organized as follows:

- Section 2 revisits the application/service perspective of UMOBILE, described in D2.1 and D2.2, and introduces more generic requirements that end up on the definition of the different UMOBILE services.

- Section 3 introduces the architecture design of the UMOBILE platform including the list of different devices that will support UMOBILE.

- Section 4 details all the modules initially presented in the architecture.

- Section 5 presents the development plan of the different UMOBILE services in the technical tasks and work packages of the UMOBILE project.

- Section 6 concludes this deliverable.

# 2 UMOBILE architecture: Application/Service perspective

In this Section, we start by identifying (Section 2.1) the different applications exploited in UMOBILE, revisiting the four main applicability scenarios detailed in D2.1 [3]: micro-blogging, emergency situation, civil protection and social routine improvement. After that, we identify (Section 2.2) the network services that UMOBILE should provide in order to support the deployment of such applications in the mentioned scenarios, and in Section 3.2 will present the set of UMOBILE service aimed at extend NDN satisfying these requirements.

# UMOBILE applicability

This section describes the applications used in each of the four applicability scenarios. Some of these applications will then be used to demonstrate the operation of the UMOBILE framework in selected use-cases.

# Micro-blogging

Users share interests in the form of tagged information where all UMOBILE users can benefit from the local information such as an interesting event, recommended place and social interaction activities.

- Recommendation (e.g. shopping, parking)

- Local News (e.g. art exhibitions; road accidents)

- Chat (e.g. music events)

# Emergency situation

UMOBILE system aims to assist users in disseminating emergency information directly via other end-user devices as well as via the UMOBILE hotspots and UAVs. Emergency services can include data uploaded by users, or data collected for other devices and disseminated by UAVs, hotspots, etc.

- Instant messaging (send info to any authorities: fire);

- Emergency channel (e.g. info about safety places)

- Chat (family; school; community)

# Civil protection

UMOBILE provides mechanisms that may assist responsible authorities in the case of challenged events. For instance, in the case of an earthquake, authorities in the affected areas can exploit data from different sources (satellite imagery, sensor-based, UAVs) to efficiently organize their efforts.

- Instant messaging (communication to home)

- Emergency channel (e.g. info about safety places)

# Social routine improvement

UMOBILE capture users’ personal data to improve their daily routine. The system shall perform simple and complex activity recognition, and learn with the users’ habits to improve and prevent aspects such as social isolation. Examples of application of social routine improvement are:

- Recommendation concerning places; restaurants; lodging provided in a non-intrusive way, based on user and usage context.

- Instant messaging over intermittent connectivity to acquaintances (e.g. large event, without knowing where one’s acquaintaces are).

# Network services

This section starts by identifying the set of services that UMOBILE must support in order to allow the applications described in Section 2.1 to be used in the project applicability scenarios, as illustrated in Table 1.

Table 1: Correspondence between UMOBILE applications and services

| Applications | Network scenario | Communication Model | Context Services | Data Services | UMOBILE services |
| --- | --- | --- | --- | --- | --- |
| Recommendation (shopping, parking) | Hotspot Opportunistic | Pull | Recommendations | Dissemination Filtering | Application sharing UMOBILE forwarding Service-prefetching |
| Local News (art exhibitions; road accidents) | Hotspot Opportunistic | Pull/Push | Affinity networks Recommendations | Dissemination Filtering | Application sharing UMOBILE forwarding Service pre-fetching |
| Chat | Hotspot Opportunistic | Pull | Affinity networks | Dissemination | UMOBILE forwarding Push services |
| Instant messaging (send info to any authorities: fire); | Opportunistic | Push | Behaviour inference | Dissemination | UMOBILE forwarding Push services |
| Emergency channel (e.g. info about safety places) | Isolated area Opportunistic | Push/Pull | Affinity networking | Dissemination Prioritisation | UMOBILE forwarding Push services Service-prefetching |

As we notice from Table 1 all applications require a data dissemination service (e.g. instant messaging), operating in Push or Pull mode.

The recommendation application used in the micro-blogging and social-routine improvement scenarios requires from the UMOBILE framework the capability to disseminate, filter data and predict users’ preferences. Recommendations are pull-based communications. News and Chat applications also require a service able to identity affinity networks aimed at filter sources of news and chat members and push-based communications. Instant messaging requires push services in opportunistic communications and can use behaviour inference to improve the data dissemination of messages.  To support an emergency channel, the UMOBILE framework also needs to provide user behaviour inference which is aimed at supporting automatic data subscription (e.g. alerts can be launched with more frequency depending on anxiety level; data dissemination can take into account the user social interaction). The different applications must be able to operate in different network scenarios; these network scenarios depend on the connectivity level of the nodes.

The different applications require different functionalities related with the network scenarios, communications models, context services or data services. Depending on this, we will have to develop different UMOBILE services aimed at supporting all these features. In the table, we see that the applications can work on different network scenarios, detailed in Section 2.2.1. The applications may require different communications model (push/pull), detailed in Section 2.2.2. They may require as well different context services (Section 2.2.3) and/or different data services (Section 2.2.4). As a result of all these requirements we will work on different extensions to NDN (UMOBILE services -Section 3.2-) that may be present in each UMOBILE mobile node (e.g. smartphone), which communicate via public hotspots, isolated access points (e.g. UAV) or by using direct wireless communications (WiFi Direct).

# Network scenarios

Deriving from the different scenarios detailed in D2.2, we extracted 3 different network context that UMOBILE architecture must cope with. These network contexts are detailed below:

- WiFi hotspots w/ or w/o back haul internet connectivity

The first case (Fig. 4) includes a network scenario where a set of users are connected to a WiFi hotspot that centralizes the communications inside a Basic Service Set (BSS). Each WiFi hotspot can be connected to the Internet, or it can be connected to another WiFi hotspot providing interconnectivity between different WiFi areas.

Figure 4 – WiFi hotspot network scenario

- Opportunistic networks based on WiFi Direct

In the second case, we focus on a network scenario where users must deal with opportunistic communication. That is, users do not have pervasive connection to the Internet, and they can have temporary encounters with other users, creating temporary connections between them. There are different cases of opportunistic contexts that we take into account in UMOBILE, as depicted in Figure 5:

- Users groups               	  			b)     Opportunistic users contacts

- Opportunistic network context with gateway

Figure 5 – Opportunistic network scenario

The first example (Fig. 5a) shows two formed groups of users connected between them using WiFi Direct connectivity. The second example (Fig. 5b) shows a set of different users that have sporadic contacts between them without forming a group between all the users. The third example (Fig. 5c) shows a set of users that can use opportunistic communications between them in order to reach a gateway, that may be co-located in the WiFi hotspot.

- Poorly connected/isolated areas

The last network context (Fig. 6) takes into account the scenario where a set of users are isolated from the network, but a mobile device interconnects them with the rest of the network using DTN capabilities. This mobile device acts as a data mule, and it can be a UAV device or any other vehicle, such as a public vehicle. Delay-tolerant forwarding can be beneficial in cases where data needs to be forwarded over vehicles, such as UAVs or emergency vehicles. This kind of connectivity can be deployed to provide communications in remote areas or in emergency cases. In addition, UMOBILE architecture also allows services to be run locally within the isolated areas in order to offload the communication over the intermittent link as well as enhance the QoS. This can be facilitated by the service migration module where the service at the mainframe can be migrated to the local access point by exploiting DTN capabilities. Nevertheless, the local access point may need to interact with the mainframe, for instance there is a new update for the operating service or local access point synchronises the usage data with the mainframe.

Figure 6 – Isolated areas network scenario

# Communication models

As shown in Table 1 most of the identified applications require a data dissemination service operating pull mode (recommendations, news, chat). Aiming to support data dissemination in a pull mode, UMOBILE adopts the concept of ICN, which enforces the receiver-oriented chunk based transport and in-network caching. While in-network caching is beneficial under mobility, UMOBILE messaging applications (emergency, instant messages [6]) are push based. Therefore, the UMOBILE framework includes a push mode data communication as a fundamental unit and leverage both push and pull modes by application context.

Based on the scenario requirements, we define both a pull- and push-based communication model to be employed by the UMOBILE platform.

- Pull-based model: This model is already supported by NDN. Clients requesting content generate and forward Interests. Content Providers (or any other node with the corresponding content cached) respond forwarding data towards the Interface they received the Interest.

- Push-based model: With the current ICN model it is possible to support sender-oriented applications, e.g. [7,9], by adapting the pull mode in order to have interests being registered in advance: e.g. I’m always interested in receiving emergency messages or messages from a pre-defined set of users. This approach has two major limitations: it requires the receiver to negotiate the data name in advance, which may not be suitable in unpredictable situations (e.g. an event); it may lead to a large overhead in UMOBILE scenarios, where nodes are mostly mobile and connectivity may be intermittent. We define several options to support a push-based model:

  - Permanent interests in PIT: This can be used to receive information from authorised authorities, such as the Fire Service, or the Civil Protection Service. In this case, each node has a corresponding Pending Interest in its PIT, leading to both local (an app running in the node) and remote interfaces (forwarding to other nodes). Specific attributes (such as location) can be employed to avoid forwarding information to non-affected areas. To do that, each PIT entry related to this type of service also includes some metadata. To avoid spamming, only authorised, authenticated messages will be forwarded in any case.

  - Server sends Interest -> Client responds with Interest -> Server responds with Information: This involves a special type of Interest that does not request data to be sent back, but rather triggers an Interest transmission from the receiving end. Implementation-wise, upon the Interest reception, a FIB entry at the client side forwards this type of Interests to local apps (when available). From this point on, a typical Pull service is initiated by the app. Modifying the Interest packet format to allow for extra fields, such as attributes or any other information regarding the advertised service, will have the advantage of allowing the receiving end to select whether he/she is interested in receiving the corresponding data. (Essentially, the initial interest should include at its naming scheme the required naming of the second interest that the client needs to send to get the pushed information)

  - Server sends Interest -> Client responds with dummy message/interest -> Server responds with information: This option can be used in scenarios where there is prefix information available towards the node receiving the information (e.g. Fire Service) but not towards the node sending the information (e.g. a tourist in the forest). In this case the server (tourist) sends an Interest towards the authorities, the authorities respond with a dummy packet that encapsulates an Interest, since responding with an Interest would necessitate finding a route back to the server (tourist), which may not be available. We modify the NDN Incoming Data pipeline so as to support Interest packets encapsulated in Data packets. Upon receiving the encapsulated Interest packet, the server application forwards the data message to the corresponding interfaces.

# Context services

As shown in Table 1 the identified applications require different context services, in order to improve and adapt the behaviour of the applications to the network context:

- Affinity networking: Affinity networking takes benefit of the networking context in order provide to users the control of the behaviour of the network directly by defining these affinity relationships and attributing specific policies to those relationships.

- Recommendations:  The recommendations service assists the users in finding and info quicker and easily, based on users’ preferences.

- Behaviour inference: No need to always access the Internet (good in large events); optimization based on wishes, needs and interests

The contextualization and behaviour inference module then integrates specific models.  For instance, a UMOBILE mobile device may present a roaming preferences model, where the intent is to consider personal preferences in terms of visited wireless networks. Or, it may contain a model that relates with the need to share data opportunistically based on frequency or volume of wireless contacts. These models are defined by the user, based on specific parameters. The identified user's context is then used, together with the collected sensing data, to infer patterns of user behaviour.

# Data services

As shown in Table 1 the identified applications require different data services in order to deliver and share data between UMOBILE nodes:

- Data prioritisation: Some applications, such as emergency services, requires prioritisation over other data types with less criticality. Data dissemination should be optimised by prioritisation rules, integrated within the information message’s name to favour spreading of the most important messages. The message replication will be limited by time and space, that is, within a certain geographic area and with specific life expectancy

- Data dissemination: This service is in charge of packet routing in a UMOBILE network. Based on the application and network context, different UMOBILE forwarding models are supported. This forwarding modes are detailed in Section 3.2. The selection of the best forwarding model to be used is done by the data dissemination service after analysis the context inherent to the data to be disseminated. SMART ROUTING MODULE???

- Data filtering: This service is in charge of gathering data about physical surroundings and social context, behaviour inference and the detection of affinity networks.

# UMOBILE architecture design

The envisioned UMOBILE architecture will efficiently operate in different network situations. Our intention is to extend the reach of ICN architectures to disconnected networks. Therefore, we intend to enhance the ICN architectural approach with an inherent tolerance to delays, disruptions and disconnections, inserting the DTN principles into the core of UMOBILE architecture. The great majority of the delay-tolerant approaches in the literature are based on the IP host-centric model, so when a node initiates a communication, its aim is to specifically reach another well-defined host.  UMOBILE architecture is an information-centric delay-tolerant communication platform based on a node architecture that unifies the various underlying protocol choices within a single architectural framework. UMOBILE integrates DTN capabilities into a single ICN architecture on top of IP enabling diversity in supported networks. In UMOBILE, we do not provide a new Internet architecture, such as Named-Data Networking (NDN) [8]. In UMOBILE we focus on the mobile domain (as depicted in Figure 6), providing a new network architecture able to interact in social/opportunistic communications and based in ICN and DTN principles, with some extensions to the wired domain in order to bring the services closer to the users, improving the QoS. In the following sections, we detail the different devices that will support the UMOBILE framework, and the set of UMOBILE services that will be developed within the UMOBILE project, and integrated in the UMOBILE architecture.

# UMOBILE devices

In the following Figure we show a summary of the UMOBILE network scenarios and the different devices that should support UMOBILE in order to provide all the services previously detailed:

Figure 7 – UMOBILE architecture

In order to provide opportunistic communications, a set of devices must support UMOBILE architecture:

- UMOBILE-enabled mobile devices: (i.e., smartphone, tablet, wearable device,etc), used to send and receive participatory data (e.g. photos, short messages) as well as opportunistic data (e.g. atmospheric pressure, temperature, noise, people anxiety levels, roaming patterns).

Figure 8 – UMOBILE end-users

- UMOBILE-enabled hotspots: able to collect and relay relevant information (e.g., alert messages, instructions from emergency authorities), host some instantiated services (using the Service Migration function) or store collected data, check its validity and perform computational functions (e.g. data fusion) to increase the value of the information to the civil authorities.

Figure 9 – UMOBILE hotspot

- UMOBILE-enabled surrogate devices: or other vehicles able to collect and relay relevant information and connect two isolated areas.

Figure 10 – UMOBILE surrogate devices

- UMOBILE-enabled gateway: In order to provide connectivity between the UMOBILE network and the Internet (IP network), we devise a UMOBILE gateway able to translate interest packets to HTTP requests and vice versa. The UMOBILE gateway can be colocated in a Public Hotspot, or a UMOBILE router. This device should have two services, one that translates UMOBILE interests into HTTP requests and another that converts HTTP GET requests into UMOBILE interests. In order to do that, we can use as starting point for the implementation of the UMOBILE proxy/gateway, we will use the HttpProxy application of the NDNx project [21].

Figure 11 – UMOBILE gateway

- UMOBILE-enabled routers: In order to provide the Service migration service and the QoS mechanisms devised in this project, we require the UMOBILE-enabled routers. The difference between UMOBILE-enable routers and Internet routers is that UMOBILE-routers are supporting the UMOBILE protocols (based on NDN) and are compatible with the UMOBILE QoS mechanisms.

Figure 12 – UMOBILE router

The UMOBILE architecture must be able to work in the different areas of action detailed in D2.1 [3]. Therefore, depending on the situation, UMOBILE architecture must adapt to the environment to achieve the best performance. So, for example, it can use opportunistic networking in order to share the information between peers in a decentralized way, offloading communications from core network. On the other hand, in remote or disrupted emergency areas, UMOBILE may use DTN techniques to propagate the information from the source to the destination. To support such scenarios, the application servers which provides UMOBILE services could be able to migrate some services from the Internet and operate locally within the UMOBILE domain. For instance, the UMOBILE services could be executed at the public hotspots (as depicted in Figure 6) in order to mitigate the network latency regarding the challenged conditions (e.g., intermittent connectivity, limited bandwidth at the backhaul). This approach is the key benefit of service migration module which can improve the QoS for the end users.

In UMOBILE, content can be named using tags/keyword, aggregating content by similarities, simplifying the naming scalability issues in the NDN approach and the search of content in the UMOBILE network. This approach is further detailed in Section 4.1.1. UMOBILE architecture must support different network technologies. UMOBILE architecture must support the ability to communicate with a wide variety of underlying protocols, implementing convergence layers for several underlying technologies, like 802.11, cellular (3G/4G), satellite, Bluetooth communication, etc. The technology used by UMOBILE for opportunistic communications will be WiFi Direct, based on 802.11 technology.

# UMOBILE services

In order to perform in all the aforementioned scenarios, being able to disseminated data and access services for all UMOBILE applicability scenarios and developing the different communication models and contextualization services, in UMOBILE project we are going to develop a set of services that will be integrated into a single architecture. The set of services are the following, and we classified them per functionalities or network abstractions:

- Application sharing: UMOBILE will provide a new application-centric information sharing framework named Keyword Based Mobile Application Sharing (KEBAPP) oriented to support and provide opportunistic computing to mobile devices (smartphones, tablets, etc.). Our approach targets scenarios where large numbers of mobile devices are co-located presenting the opportunity for localised collective information exchange, decoupled from Internet-access. In KEBAPP, we propose the creation and use of 802.11 broadcast domains for the support of particular applications i.e., KEBAPP-enabled hosts or APs advertise one or more Basic Service Set(s) (BSSs) for the support of one or more application(s). The creation of application-specific BSSs aims at enabling mobile devices to connect only when their counterparts support the same application and/or namespace. The advertising AP or host, through a WiFi Direct Group, acts as a mediator to connect different users willing to share the same application in a single broadcast domain. In the case of APs, functionalities such as access control, association, encryption, etc., can be supported without imposing computation and/or battery overheads to mobile devices. In this context, KEBAPP employs application-centrism to facilitate/enable (i) the exchange of processed information, in contrast to merely static content, and (ii) the discovery and delivery of information partially matching user interests (data filtering).

- UMOBILE forwarding: The following set of forwarding mechanisms are going to be developed within the UMOBILE architecture:

  - NDNopp: In order to perform opportunistic forwarding based on users’ interests and their dynamic social behaviour, NDN-Opp includes some changes in relation to NDN in order to enable social-based information-centric routing over dynamic wireless networks. Based on the current specification of NDN, NDN-Opp provides a novel routing engine able of supporting opportunistic communications, and a forwarding engine that makes use of the existing Best Route forward strategy. NDN-Opp is the first attend to extend the NDN network towards opportunistic wireless networks, which: do not assume the existence of a communication path between any pair of nodes at any moment in time; exploit any communication opportunity between mobile devices to forward data towards receivers. To accommodate the intermittent nature of wireless connectivity, NDN-Opp introduces the concept of virtual faces. Changes to the current specification of NDN-Opp are expected in order to better align the proposed solution to the NDN framework, aiming to ensure an easy integration of UMOBILE networks in the existing NDN wired infrastructure.

  - DTN: The main goal of UMOBILE project is the development of a networking architecture that supports both information centricity and delay-tolerance. To achieve that, we will exploit not only by providing delay tolerant communications natively using the NDN platform but also working on an IBR-DTN implementation. This IBR-DTN implementation will provide opportunistic communications through DTN tunneling in case of opportunistic communications when delay-tolerant transmission of data is deemed the most suitable alternative.

  - OOCD: We will investigate a new scheme named Opportunistic Off-path content discovery (OOCD) the retrieval of content in disruptive, fragmented networks cases in NDN. To resolve and fetch content when the origin is not available due to fragmentation, we exploit content cached both in in-network caches and in end-users' devices. We introduce an extra Interest management routing table, which we call the “Satisfied Interest Table” (SIT) and which points to the direction of already satisfied Interests. This way, upon failure of links/nodes towards the content origin, the SIT table is redirecting Interests towards caches and end-users that have recently received the requested content. We believe that network management for disruptive environments should take advantage of information-centricity, instead of focusing only on protocol-specific path recovery routing.

  - NREP: In the immediate aftermath of a natural disaster, network infrastructure is likely to have suffered severe damages that challenge normal communications. In addition to that, traffic substantially increases as a result of people attempting to get in touch with friends, relatives or the rescue teams. To address such requirements of a challenged network, we are going to propose a mobile name-based replication system for disaster cases based on Name-based REPlication priorities (NREP), where replication is optimised by prioritisation rules, integrated within the information message’s name to favour spreading of the most important messages. The message replication will be limited by time and space, that is, within a certain geographic area and with specific life expectancy

- Push services: Based on the scenario requirements, UMOBILE platform requires both pull and push-based communication models. The pull-based model is inherently supported by NDN. Clients can request the desired content by issuing Interests with name prefix. Content Providers or any other NDN node with the corresponding content cached can respond forwarding data towards the Interface regarding information in the PIT. However, we can adopt several models of push-based communication by using default Interest/Data exchange of NDN architecture, firstly introduced in Section 2.2.1. In the following deliverable (D3.1) we will give more detail about the solutions adopted by the UMOBILE platform in order to provide Push services to the UMOBILE applications.

- QoS: In order to ensure QoS to end-users, we present two complementary mechanisms:

  - Service pre-fetching: Service pre-fetching is one of the means that the UMOBILE platform uses to ensure that the services at delivered with the expected QoS to the end users, or we can provide local services where there is no Internet connectivity, by means of the service migration platform. For example, service migration guarantees that a given service is resilient and it observes, latency, throughput and other QoS requirements. Thus the UMOBILE platform implements mechanisms that can help network operators migrate services to different places of its network infrastructure (normally the network edges) in order to honour the expected QoS as well as accommodate service reachability in challenged networks [19]. As mentioned in D2.1 (use case scenarios), UMOBILE needs to support various challenged scenarios such as aftermath of disasters or in rural/remote network deployments or networks with limited backhaul capacity. Such challenged environments pose several challenges such as increased latency, intermittent connectivity etc.  To address these challenges the UMOBILE architecture includes a service migration platform implemented on the basis of the latest advances in lightweight operating systems such as Docker containers [add citation]. The key idea is to deploy and migrate service instances to the network edge. Within a local network, service migration modules utilise a name based routing/forwarding strategy providing the benefits of ICN. Within the UMOBILE architecture, we will utilise the NDN abstractions to carry out the name based routing/forwarding strategies. We also envision that our service migration module could be integrated with DTN to provide resilient access to services during periods of intermittent connectivity. Ideally, service migration should be transparent to the end users, in the sense that they do not need to know the location of the service replica that is responding to their requests. For this transparency to be possible, the service replicas need to be migrated before in anticipation of the end users’ needs (pre-fetched).  To address this requirement, the UMOBILE architecture includes a decision engine.  The decision engine is a software component instrumented with the necessary logics to i) collect monitored metrics about parameters that impact the QoS and ii) determine when and where a service should be migrated and iii) migrate the service. Typical examples of parameters that are taken into account by the decision engine are the number of users interested in the service, load inflicted on existing replicas, current network traffic, migration cost in terms of time and traffic, potential benefit in terms of latency, etc.

  - INRPP: Within the UMOBILE project, we aim to design and evaluate the In-Network Resource Pooling Protocol (INRPP), which pools bandwidth and in-network cache resources in a novel congestion control framework to reach global fairness and local stability. Taking profit of the hop-by-hop design and the caching capabilities inherent in the NDN networks, or adding caches (i.e., temporary storage) and breaking the end-to-end principle, we argue that the demand factor can be tamed. Given this functionality of in-network storage, INRPP comprises three different modes of operation: push mode, store and detour mode or backpressure mode.

- Contextualization services

  - PerSense Mobile Light: In the context of UMOBILE, PerSense Mobile Light is a service that it is being developed to assist in performing network contextualization. Currently, PerSense Mobile Light captures information concerning a user’s affinity network (contacts derived from Wi-Fi Direct and Bluetooth) as well as concerning roaming habits, over time and space (Wi-Fi). In a future version, PerSense Mobile Light shall collect data on user behavior, derived from additional sensors – the sociability forecasting module.

# Network requirements

SHOULD WE ADD THE COMPLETE LIST OF REQUIREMENTS, OR IS IT BETTER TO ONLY ADD A REFERENCE TO IT?

In this section we give some details about how the network requirements detailed in D2.2 [add citation] are going to be satisfied by the different UMOBILE services previously presented. In D3.1 we will give more detail about how these network requirements are implemented.

These are the network requirements listed in D2.2 that we will point how the different UMOBILE services will deal with them. Requirements marked in red are those requirements that are classified as SHOULD, and black requirements are those classified as MUST requirements (Note that SHOULD requirements R-18, R-21, R-23 and R-24 and MAY requirements are out of the scope of this document and will be investigated in further deliverables -D3.4 and D3.2-):

- Pre-fetched services: R-8, R-13, R-16, R-19, R-22.

The pre-fetched services are aimed at bringing services at the edge of the network, avoiding the dependency from the Internet connectivity and providing local services to users, without requiring end-to-end connectivity.

- Application sharing: R-8, R-13, R-14, R-16, R-22.

Application sharing services are aimed at providing application sharing capabilities through D2D communications between end-users. Users can access to services but provided by other end-users when the access to Internet is limited, complementing the pre-fetched services at the edge of the network.

- UMOBILE forwarding:

The UMOBILE forwarding mechanisms are aimed at satisfying those requirements related with i) providing forwarding mechanisms based on users interests -inherent in NDN-, ii) providing social-based opportunistic forwarding, iii) providing forwarding mechanisms that support data prioritisation for emergency services, iv) providing new forwarding mechanisms compatible with a DTN logic face and v) providing a new opportunistic off-path contend discovery mechanism (OOCD) aimed at improving content localisation in fragmented networks. The list of requirements of D2.2 related with the UMOBILE forwarding mechanisms are:

  - NDN: R-3, R-7, R-11, R-15.

  - NDN-Opp: R-1, R-2, R-3.

  - NREP:  R-2, R-3, R-9, R-10.

  - DTN: R-2, R-3.

  - OOCD: R-2, R-3.

- Context services: R-3, R4, R-14, R-15, R-17.

The context services are aimed at satisfying those services related with improving the social routine and providing information about user’s interest and context. Context services provide recommendation services, affinity networking and behaviour inference.

- New devices: R-6, R-12, R-20.

In the UMOBILE project we devise new devices (Section 3.1) aimed at providing services to isolated users. These new devices are aimed at satisfying network requirements related with data and services availability in remote areas.

# UMOBILE modules

In this Section we describe the different modules that are part of the UMOBILE architecture, shown in Figure 2 and 3 in Section 1.

# NDN plane

The UMOBILE architecture is based on the current NDN architecture and will reuse most of its components, adding new ones to provide the features and network requirements previously specified. These NDN components that will be reused by the UMOBILE platform are: communications based on interests, content store, Pending Interest Table (PIT), Forward Information Base (FIB), caching policies and forwarding strategies. The naming used in NDN will be modified in order to provide more flexibility, required in opportunistic communications where the lack of infrastructure requires more flexibility in terms of the content that can be found in the different nodes.

# Naming

The discovery and invocation of services/applications in the networking vicinity of a user build on a naming scheme that enables the fine-grained description of the desired information.

To this end, UMOBILE builds on the observation that mobile computing is largely application-centric, in the sense that users tend to access information using purpose-built applications. Application-centricity presents a series of important characteristics:

- Applications inherently support the structuring of the name space within their semantic context.  In turn, instances of the same (or similar) application can share the same name space in describing the related information e.g., categories in a news application.

- Applications are inherently used for computation, enabling the (lightweight) processing of content/information e.g., searching, sorting data or computing a route.

Taking these features into account, UMOBILE names are composed of two main parts (see Fig. 12):

- Fixed Hierarchical Part. It follows the hierarchical naming scheme of NDN and its purpose is to guarantee compatibility between instances of the same or different services/applications.  Application developers can define their own hierarchical name spaces, enabling the communication between different instances of the application e.g., /NewsApp/politics/international. Further enabling communication between different applications goes through naming conventions that can build on the currently followed categorization of applications in popular application markets e.g., extending “/Travel & Local/” in Android App Market, to “/Travel & Local/Car Rentals”.

Moreover, application developers can also define suffixes corresponding to specific functionalities within their applications (in addition  to static content), enabling this way the sharing of computation e.g., the name /MyTravelAdvisor/Top10Restaurants is used to  identify the list of the top-10 restaurants in a certain area.  The Hashtag part of the name, described below, is used to define the sorting criteria and the selected area.

According to our initial design this part of the name will have to be an exact match in order for the request to be formed. That said, however, it will be difficult for users to imagine the different parts of the name that the application developer has defined.

- Hashtags. The second part of the name comprises of hashtag-like keywords, which the application developer can add to the application.  The exact semantics of the hashtags depending on whether the fixed hierarchical part of the name corresponds to static content or an application function(ality).  In the former case, these keywords are used to semantically annotate the static content.   This feature enables the partial matching of requests with available cache or routing/forwarding entries i.e., given an exact match in the fixed hierarchical part of the name, hashtags can be used to support approximate matching enabling the search of information in other devices.

When the fixed part of the name identifies a certain application function(ality), the hashtag part of the name enables the passing of adequate parameters.  In the aforementioned example of the MyTravelAdvisor application, the complete name included in a user request can have the fixed hierarchical part /MyTravelAdvisor/Top10Restaurants and the hashtags \#userrating, \#areaY, \#indian indicating that the user is interested in the top-10 of the indian restaurants in areaY, according to users’ ratings.  The submission of hashtag values is guided by the application GUI and can include both predefined value ranges e.g., the sorting criteria for the top-10 restaurants, and free text fields e.g., a user requests /MyNewsApp/politics/search #Syria #negotiations to use the search function of MyNewsApp in order to find anything related to negotiations for Syria. In another example, an application following the naming conventions of the app market place issues a request for the name /news/politics/search #Syria #negotiations, which can match any application supporting a search function.

Figure 13 - UMOBILE keyword-based naming scheme

We believe that the naming-scheme presented above can be rather efficient in narrowing down the range of the services to be executed and therefore, the results that will be returned to the user.

Below we include example naming schemes for the proposed applications in Section 2.2:

- Recommendations:

A user is walking on Oxford Street in London and wants to but men’s clothes that are on sale.

Fixed Hierarchical Part: 	/ShoppingApp/Clothes/Men

Hashtags:			\#London \#OxfordStreet \#Sales

- Chat:

A user would like to participate in a chat with other parents of the same class of their children school, in case of emergency.

Fixed Hierarchical Part: 	/ EmergencyApp /Chat

Hashtags:			\#SchoolName \#3rdGrade

- Instant messaging:

In this scenario, a user wanst to establish communications with any fire station in central London to provide information about a fire.

Fixed Hierarchical Part: 	/ EmergencyApp /InstantMessaging

Hashtags:			\#CentralLondon \#FireStation

- Emergency channel:

In this scenario, a user knows that a fire has broken out within the university campus in Xanthi and is looking for an evacuation plan.

Fixed Hierarchical Part: 	/EmergencyApp/Xanthi/Fire

Hashtags:			\#UniversityCampus \#EvacuationPlan

- Local News:

A user is willing to find gigs and live music in central London.

Fixed Hierarchical Part: 	/NewsApp/arts/exhibitions

Hashtags:			\#CentralLondon \#Music \#Today

# UMOBILE forwarding strategies

In order to provide more features and opportunistic capabilities to the NDN architecture, in UMOBILE we need to add different strategies that the ones provided by NDN, that are best route, broadcast, client control and NCC [22].

The core of our information-centric communication model lies in the usage of Interests issued by clients (pull-based model) or content providers (push-based model). In order for these Interests to be successfully delivered, a decision must be made on whether, when and to whom an Interest should be forwarded. Given the diverse application requirements, along with the varying networking conditions that may span from continuous high-speed connectivity to intermittent disruptive communications, a collection of different forwarding strategies needs to be supported by the UMOBILE architecture.

In particular, we envision a per-namespace selection of the forwarding strategy and we divide the forwarding strategies that UMOBILE will support into four main categories:

- Existing NDN forwarding strategies, such as best route, broadcast, client control and NCC.

- Delay-Tolerant NDN strategies, which are similar to the existing NDN forwarding strategies but support increased timer expiration values.

- DTN tunnelling strategies, where opportunistic forwarding is exploited for DTN nodes that interconnect ICN nodes.

- Opportunistic forwarding based on users’ interests and their dynamic social behaviour.

All basic forwarding decisions are made in the Forwarding module, a central part of the UMOBILE platform. In particular, the Forwarding module is responsible for selecting the optimal way to forward packets, based on the following inputs:

- Usage context (e.g. packet priority, application delay-tolerance, user delay-tolerance).

- Network context (e.g. WiFi Direct connectivity, UAV connectivity).

- User context (social information).

In the following, we describe the forwarding strategies that the UMOBILE platform will support.

Typical NDN

Interoperability between UMOBILE and NDN necessitates that the typical NDN forwarding strategies are also supported. In particular, UMOBILE will include the following NDN strategies:

- Best Route: This strategy forwards Interests to the interface with lowest routing cost.

- Broadcast: The broadcast strategy forwards Interests to all eligible interfaces.

- Client Control: This strategy allows a local application to choose the outgoing face of each Interest.

- Ncc[22]: This strategy, initially implemented for CCNx, is capable of employing multiple Interfaces to forward Interests, based on a prediction function.

In the framework of the UMOBILE project, we devise a new set of forwarding strategies as a means to support scenarios involving long end-to-end delays but short hop-by-hop links. Typical use cases under this category are transmissions involving data ferries (e.g., UAVs). More specifically, we provide delay-tolerant characteristics to the strategies described in 4.3.1.2.1 by extending specific timers and modifying Interest and Data packet pipelines to exploit Interfaces that become available in a future moment.

DTN

This forwarding strategy effectively connects NDN nodes over DTN links. Typical use cases for this type of connectivity are opportunistic networks (e.g., festival or emergency communications). Implementation-wise, NDN faces are translated to DTN hostnames, so that each NDN-DTN Face corresponds to a single DTN node within the DTN island. This way, the NDN forwarding engine actually delivers the Interest and Data packets to the DTN protocol. Then, DTN forwarding strategy is applied, and attempts to forward the Data/Interest packet to the specified DTN host. The starting point for DTN forwarding strategies is Tecnalia’s HURRywalla implementation [15].

KEBAPP

This forwarding strategy is aimed at supporting the KEBAPP functionalities in order to enable application sharing. The details of this new forwarding strategy will be detailed in D3.1.

OOCD

This forwarding strategy is aimed at forwarding the content not only from the source of the content, but also using the NDN extension the OOCD provides, to discover other users that recently downloaded the same content, and are willing to offer the same to content to the requesting user, in case that network fragmentation does not allow to retrieve the content from the source of it.

NREP

NREP forwarding strategy is aimed at support the Name-based REPlication priorities (NREP) approach, where replication is optimised by prioritisation rules, integrated within the information message’s name to favour spreading of the most important messages. The message replication will be limited by time and space, that is, within a certain geographic area and with specific life expectancy, enabling the emergency services prioritisation in disaster cases.

NDNOpp: Social-aware data-centric forwarding strategy

The current specification of NDN-Opp aims to handle the dynamics of opportunistic wireless networks by forwarding interest packets towards best neighbours, which are selected based on their probability to deliver packets to a node carrying the interested data. This means that the current specification of NDN-Opp makes use of:

- The existing best route forwarding strategy to deliver interest packets. In this case the cost is related to the social weight computed by the social proximity module.

- The NDN “breadcrumb” approach to deliver data packets based on the information stored on the PIT.

The cost of each name prefix (stored in the FIB) is the value of the social weight compute for the best next hop, as computed by the social proximity module.

In summary, NDN-Opp encompasses the following novel contributions to the NDN framework in order to allow its operation over opportunistic networks:

- Novel social-aware routing engine, which is able to implement different algorithms able to select best routes based on the social weights computed from a social proximity module.

- Novel forwarder able to support opportunistic communications.

- Integrates the concept of Long Lived Interest in order to support push communication model.

  - Users manifest their interest in getting data from some type of application (e.g. short messaging) or from some specific node.

  - Interests persist in the network for as long as established by the Interest source by setting a Long Lived Interest parameter.

  - While Interests persist in the network, data can flow towards the established intermittent path.

- Integrates the concept of virtual intermittent face, which can be in two states (ON/OFF).

  - Virtual interfaces include queues of interests and data, aiming to speed up transmission over short-lived wireless links.

- Makes use of:

  - Best route forwarding strategy to handle Interest packets based on social weights;

  - The default “breadcrumb” approach to forward data..

- Two new fields in the PIT and FIB:

  - Social Weights in the FIB

  - Long Lived Interest (TTL) in the PIT.

- Novel NDN face: Wi-Fi direct.

- Updated Bluetooth face (we need to gather info from neighbour, and not to send/recv data).

The forwarder follows the normal NDN operation by dispatching data packets based on the information stored in the PIT, and dispatching Interest packets based on the information stored in the FIB. To forward Interest packets, the best route forwarding strategy is used, in which the FIB stores the cost associated to each name prefix

# Social routing

A NDN-Opp node goes over a main set of operations when it gets in contact with another mobile NDN-Opp node. The social routing module includes the social proximity module and the routing engine, required to execute the set of operations.

- Social Proximity Module: This module is the responsible for the computation of social weights based on information collected about neighbour devices, in order to allow the social-aware routing engine to compute best routes. A node updates its social weight towards each of the names carried by a neighbour node.

- Social-aware Routing Engine: This module is responsible to select the best neighbour to reach data related to a certain name prefix. For this propose the routing engine can run any type of social-aware opportunistic routing algorithm [2]. The first algorithm used by NDN-Opp is the Time-Evolving Contact Duration approach [3] used by the dLife protocol [4]. A new social-aware routing protocol able to generate less volatile connectivity graphs is being developed.

The Social Proximity Module is able to compute social weights to characterize the link between neighbours. The operation of the social proximity module is based on a social weight measurer, a social weight repository and a gatherer of information about social weight and data carried by neighbour nodes:

- Social Weight Measurer (SWM) – responsible for keeping track of the contact duration between the current and encountered nodes. SWM maps this duration to the name prefixes carried by the encountered nodes. Based on that, this component computes the level of social interaction of the current node towards the name prefixes carried by encountered peers. This social weight determines how good the encountered node is to reach nodes that carry data related to such name prefixes. The social weight computation takes place at every hour as to allow for a better view of the dynamic social behaviour of users.

- Social Weight Repository (SWR) – responsible for storing the list of name prefixes that the current node comes across (obtained upon encountering a peer). SWR holds the name prefix, time it has been first encountered, and the cumulative duration of within a specific hour. In the case, the current node still 'sees' such name prefix (i.e., respective peer node is still in the vicinity), the time of first encounter is updated and contact duration is accounted by SWM for the new hour.

- Social Weight and Carried Data Gatherer (SWCDG) – responsible for obtaining the list of name prefixes and social weight towards them of encountered node. This element is also responsible for obtaining information concerning the content carried by encountered nodes.

The computation of social weights is triggered when a specific face notifies the SWM of the presence of a neighbour node. The SMW creates entries for this encountered node in the SWR considering the name prefix information obtained by the SWDCG. The SWM keeps track of the contact duration between current and encountered nodes and updates the SWR accordingly. Since nodes (i.e., their users) present different patterns of behaviour in different time periods, the computation of social weights takes place in an hourly fashion. Information obtained by the SWCDG from the encountered node is used to populate PIT and the content store.

The development of the NDN-Opp framework will be done based on the integration of the NDN-Opp with the Content Information Module of the UMOBILE framework and on the development of a novel routing scheme. The major action points are:

- Interface with the UMOBILE Context Information Module:

  - The interface of NDN-Opp with the Context Information Module, illustrated in Fig 11, will be done in the context of WP5.

  - The collection of context information about neighbour nodes, done currently by scanning the Bluetooth face, should be done also by scanning the Wi-Fi direct face. The goal is to allow the collection of neighbour information to be done over the same wireless range as the one used to exchange data. Subsequent evaluation about the performance of both approaches will be done based on a UMOBILE testbed.

- Novel opportunistic routing mechanism. Currently NDN-Opp makes use of the social-aware algorithm used by existing routing approaches (dLife/SCORP). We will investigate an alternative proposal for the NDN-Opp framework based on novel strategies to forwarding interest packets in dynamic networks. In a first stage we will keep using the NDN default “breadcrumb” strategy to forward data. In a second stage we will investigate a more intelligent method also to forward data, aiming to exploit any contact with a neighbour node able of forwarding data towards the receiver(s), and not just the wireless contact with the node that previously sent the Interest packet (which in the approach followed in the default NDN “breadcrumb” approach). The performance of both alternatives will be assessed in real scenarios.

While the NDN-Opp framework is still being specified, COPELABS already implemented the proposed framework in a monolithic scheme: SOCIO. This is, SOCIO implements the current routing engine and social proximity module proposed for NDN-Opp outside the NDN framework. The goal was to allow a faster evaluation of the proposed forwarding and routing engines, which was feasible since the specificities of the NDN framework were not considered. This step-by-step approach allowed us a faster detection of potential improvements, such as the need to develop a novel opportunistic routing mechanism that would be more aligned with the operation of NDN. D3.1 provides a high level view of the first draft of the NDN-Opp framework and  a detailed description of the SOCIO implementation in Android.

# Context manager

The context plane takes care of the collection, resolution, and storage of the context.  The context can be related to the usage or user context. The usage context can be retrieved using metadata included in data packets, in order to differentiate the characteristics of different services. This information must be used to adapt the routing options to the requirements of each service.  The user context can be retrieved based on local connectivity as well as device usage, mainly to be used for the social forwarding engine (Section 4.2). The network-condition context can be retrieved and used to adapt the routing decisions to the area of action (e.g. crowded urban area or sparse network) and the condition of each network interface (connected/disconnected, packet loss, etc). This could also differentiate the mode of operation between e.g., connected and disconnected mode. This information can be used to decide what is the best network interface for sending a certain packet.

# Usage context

In order to differentiate the network requirement for each one of the UMOBILE applications, we need to retrieve information from the application data. To this end, the usage context module is responsible of retrieving this information from the application data using metadata included in data packets. This information must be used to adapt the forwarding options to the requirements of each service (e.g. providing more priority to some services). Moreover, services are often faced with stringent requirements in terms of performance, delay, and service uptime. On the other hand, little is known about the performance of applications in the network, for instance the response time variation is induced by network connectivity and traffic load.  In response to these concerns, there has been significant interest in the usage context how the applications and services behave at the end users’ device. The usage context information accommodates the forwarding strategies to be adaptive to the various network conditions and service requirements. This significantly improve the content and service delivery such that the service requirement can be satisfied.

To retrieve the usage context, UMOBILE applications are integrated with the service monitoring where usage context (e.g., service response) is collected automatically from the users’ devices (e.g., smartphone). Consequently, this information will be uploaded to the UMOBILE server through the nearest correspondence nodes (e.g. local WiFi access point, other users’ device with WiFi Direct).

Since, we will use NDN as a reference for our ICN approach, for this purpose, we will use the already defined in the NDN data packet MetaInfo field (Fig. 5).

# User context

User context is captured based on local connectivity as well as device usage. A social routine inference service inserted into the UMOBILE is a non-intrusive service which resides on the end-user device.  By non-intrusive it is meant that this service takes advantage of the natural networking footprint that is overhead by devices, be it via WiFi, Bluetooth, as well as any other means (e.g. LTE Direct). In UMOBILE, however, the effort will be focused on short-range wireless in the form of WiFi and WiFi Direct.

The operation of this module starts by having sensing data begin collected by mobile devices, by means of a cooperative sensing functionality. Although the proposed cooperative sensing middleware allows the configuration of any sensor available in a mobile device, the proposed setting is able of collecting raw sensing data about users' roaming and relative location, by using the WiFi interface; about users' social interactions, by using the WiFi and Bluetooth interfaces, and about users' behavior, e.g. by using data concerning device usage.

Such “smart” captured data (in contrast to raw data) is then made available to a contextualization and behaviour inference module as well as to the routing/data sharing module. The data shall reside on the end-user devices only, so the UMOBILE system obeys to the most recent EU rules concerning data privacy.

The contextualization and behaviour inference module then integrates specific models.  For instance, a UMOBILE mobile device may present a roaming preferences model, where the intent is to consider personal preferences in terms of visited wireless networks. Or, it may contain a model that relates with the need to share data opportunistically based on frequency or volume of wireless contacts. These models are defined by the user, based on specific parameters. The identified user's context is then used, together with the collected sensing data, to infer patterns of user behaviour.

The inference process may be distributed among personal devices or may also include cloud computing entities, depending on the amount of data to be analysed, the required learning algorithms, as well as the delay tolerance and privacy levels of applications. Similar distribution of computational effort may be needed to adjust contextualization modelling, by considering quantifiable social parameters, and by adjusting them to the roaming dynamics that can characterize user’s behaviour with an adequate level of assurance. Some aspects that are considered in the notion of user context are:

- Social trust circles. These correspond to networks of devices owned and carried by users that share affinities, wishes, or interests. Trust circle computation assist in collective inference, derived not only from physical proximity, as well as from social proximity. Examples of trust circles are groups of friends; familiar strangers interested in a specific event.

- Social roaming footprint. The footprint that both individual and collective users exhibit when roaming around. Derived from wireless connectivity, it incorporates aspects such as duration of visits; common local places and paths.

- Personalized recommendations. Recommendations are filtered by the devices based on the personal and individual affinities of each user.

PerSense Mobile Light will be developed in the UMOBILE project to provide this contextual information to other modules that require it (such as the social routing module) but also to provide this contextual information to applications.

# Connectivity manager

Through their mobile devices, users are presented with a multitude of opportunities to access information available in their networking vicinity in the form of opportunistically cached, pre-fetched/downloaded or locally generated content and/or services/applications providing static or dynamically generated content. Depending on the networking environment, information can reside at a wide range of accessible network locations and application level user dynamics, as well as any explicit differentiation of accessible services and/or content, contribute to the expectation of information diversity in these locations. In turn, awareness of the availability (or expectancy) of information in the networking environment can lead to connectivity decisions that reflect user interests, thus facilitating or even enabling the otherwise impossible access to the desired information and creating or removing the logical faces in the NDN plane depending on the connectivity situation of the end-user.

For this reason, we propose that UMOBILE must enable information-centric connectivity, using a new module called Connectivity Manager, aimed at discovering the networks enabling and facilitating access to the desired information. In essence, information-awareness is introduced at the link layer, supporting connectivity decisions per wireless network interface. This way, it is expected to enhance user experience as information-centric connectivity decisions bring the user closer to the desired information, reducing latencies, along with network traffic.

In order to provide information-centric connectivity several options are available for WiFi interfaces. The first one is the Generic Advertisement Service (GAS) of IEEE 802.11u [16], which specifies a frame format and exchange process. The recently announced WiFi Neighbour Awareness Networking (NAN) protocol [17] also further supports a low energy consumption device discovery mechanism enhanced with publish/subscribe primitives that can serve the same purpose.

Information availability is further expected to improve when a connectivity decision either leads to the desired information or not e.g., accessing photos in the aforementioned example. Note that in this context, these benefits come without the currently imposed need to search for information upon the time and energy consuming network association process. This comes in sharp contrast to a substantial body of work on service discovery, which, in most cases, assumes the establishment of connectivity between participating devices, before any service discovery protocol is employed (e.g., Jini, UPnP).

# INRPP - Flow control

Given the dynamic nature of the considered use-cases, UMOBILE does not rely on the existence of end-to-end flows between communicating parties. Thus, the concept of flowlets is rather interesting, to provide applications with different quality of service mechanisms to allow satisfactory levels for the content being exchanged.

This functional block oversees how flowlets are started between the involved entities as to allow better, QoS-based data exchange and provide special support to in-network caching. It may provide QoS on-demand, by understanding the needs for QoS and making sure the content reaches its recipients respecting the desired QoS levels as much as possible.

However, we consider flow control out of the scope of this deliverable, and we will detail the flow control for UMOBILE in D4.1 and D4.2, with a detailed specification of the proposed rate-regulation scheme.

# Service pre-fetching

Service pre-fetching, aimed at providing local services where there is no Internet connectivity, or the Internet connectivity is intermittent, by means of the service migration platform requires the different modules that are described below:

# Service manager

The Service Manager is responsible for deciding what services to migrate and replicate, when and where to deploy them. For example, it might decide to migrate serviceA to Service Repository3. At the heart of the Service Manager is a Decision Engine. The decision engine makes decision about service migration on the basis of the QoS required by each service and monitored parameters that impact their QoS. The QoS requirements are provided by the owner of the services as deployment descriptors with QoS constraints.  Similarly, the monitored metrics are provided by a monitoring component.

# Monitoring

The monitoring component is responsible for collecting metrics about parameters that impact the QoS and of interest to the Service Manager. It can be realised as a set of monitors deployed to collect metrics about the status of the parameters of interest.  Typical examples of QoS parameters are the number of users requesting a given service, load inflicted on existing replicas, performance of replicas, resources of the computer executing the replicas (memory, CPU, disk), network traffic, latency, and so on.

# Service repository

The Service Repository is responsible for storing and executing services for the benefit of the end users. Conceptually, it can be implemented as computer with storage and processing facilities, such a Raspberry Pi. The Service Repository is capable and willing to receive compressed images of services from the Service Manager, uncompress, execute them and grant access to end users.

# UMOBILE platform WP planning

The UMOBILE services previously specified, as extensions of NDN to provide opportunistic and social communications, are going to be developed in different tasks of the UMOBILE project. These tasks are not all of them part of the WP3 -System and node architecture Development-, on which the UMOBILE platform architecture has been designed. Some of the tasks are part of the WP4 -Services enablement- as well. In the next figure it can be identified what parts of the UMOBILE platform will be developed in the WP3 and what parts of the platform will be developed in WP4, and in which tasks.

Figure 15 – WP Planning

- NDN-Opp: WP3 – Task 3.3:  Smart routing based on social interaction approaches (D3.1, D3.2)

- IBR-DTN: WP3 – Task 3.1: DTN overlay design and convergence layers for underlying protocols (D3.1, D3.2).

- KEBAPP: WP3 – Task 3.2:  Providing service-abstraction to applications through content-centric approaches (D3.1, D3.2).

- OOCD: WP3 – Task 3.2: Providing service-abstraction to applications through content-centric approaches (D3.1, D3.2).

- Push services: WP3 – Task3.2: Providing service-abstraction to applications through content-centric approaches (D3.1, D3.2).

- INRPP: WP4 – Task 4.1: Providing different levels of QoS and flow control. (D4.1, D4.2).

- Service migration platform: WP3-Task3.2:  Providing the mechanisms for Dockerizing and migrating services. (D3.1, D3.2).

- PerSense Light: WP4 – Task 4.2: Data collection and contextual inference (D4.2).

- NREP: WP4 – Task 4.3: Name-Based Replication Priorities (D4.3).

- QoS Decision Engine: WP4-Task 4.1: Providing the algorithms to determine what services to migrate, when and where (D4.4).

# Conclusion

In this document, we defined an initial approach of the UMOBILE architecture. The proposed architecture aims to deal with the different areas of action defined in the document D2.1 for the UMOBILE project and tries to integrate the DTN and ICN paradigm in a single architecture able to perform in any of the scenarios defined. This document will be complemented by the low-level design of deliverable D3.1 and D3.2, and the final version of the high-level definition of the UMOBILE architecture in the deliverable D3.4.

# References

[1] V. Cerf, S. Burleigh, A. Hooke, L. Torgerson, R. Durst, K. Scott,K. Fall, H. Weiss, “Delay-Tolerant Networking Architecture”, IETF RFC 4838, April 2007

[2] George Xylomenos, Christopher N. Ververidis, Vasilios A. Siris, Nikos Fotiou, Christos Tsilopoulos, Xenofon Vasilakos, Konstantinos V. Katsaros, and George C. Polyzos, A Survey of Information-Centric Networking Research. Communications Surveys Tutorials, IEEE, 16(2):, Second 2014.

[3] UMOBILE Project, “D.2.1 - End-User Requirements Report”, June 2015

[4] K. Scott and S. Burleigh. Bundle Protocol Specification. RFC 5050 (Experimental), 2007.

[5] G. Tyson, J. Bigham, and E. Bodanese, “Towards an information-centric delay-tolerant network," in Proceedings of IEEE INFOCOM Workshop on Emerging Design Choices in Name-Oriented Networking (NOMEN), 2013.

[6] I.Psaras, L. Saino, M. Arumaithurai, K. K. Ramakrishnan and G. Pavlou, Name-‐ Based Replication Priorities in Disaster Cases., 2nd Workshop on Name Oriented Mobility (NOM 2014) in conjunction with the IEEE INFOCOM, Toronto, Canada, 2014.

[7] Van Jacobson, Diana K. Smetters, Nicholas H. Briggs	, Michael F. Plass, Paul Stewart, James D. Thornton, Rebecca L. Braynard, “VoCCN: voice-over content-centric networks”,  in Proc. of international workshop on re-architecting the internet, Rome, Italy, December 2009.

[8] Networking Named Content by V. Jacobson, D. K. Smetters, J. D. Thornton, M. F. Plass, N.H. Briggs, R.L. Braynard. CoNEXT 2009, Rome, December, 2009.

[9] C. Tsilopoulos and G. Xylomenos, “Supporting Diverse Traffic Types in Information Centric Networks, ” in Proceedings of the ACM SIGCOMM workshop on Information-centric networking, Toronto, Canada, Aug. 2011, pp. 13.

[10] W. Moreira and P. Mendes,  "Pervasive data sharing as an enabler for mobile citizen sensing systems." Communications Magazine, IEEE 53.10 (2015): 164-170.

[11] W. Moreira and P. Mendes, “Social-aware opportunistic routing protocol based on users interactions and interests,” in Ad Hoc Networks, ser. Lecture Notes of the Institute for Computer Sciences, Social Informatics and Telecommunications Engineering, M. H. Sherif, A. Mellouk, J. Li, and P. Bellavista, Eds. Springer International Publishing, 2014, vol. 129, pp. 100–115.

[12] W. Moreira, P. Mendes, and S. Sargento, “Opportunistic routing based on daily routines,” in Proceedings of WoWMoM, June 2012, pp. 1–6.

[13] Afanasyev, Alexander, et al. NFD developers guide. Technical Report NDN-0021, NDN Project, 2014.

[14] Zhenkai Zhu and Alexander Afanasyev, “Let’s ChronoSync: Decentralized Dataset State Synchronization in Named Data Networking” in Proc. of IEEE ICNP, Goettingen, Germany, October 2013

[15] Susana Pérez-Sánchez, Jose Maria Urteaga Cabero, Iñigo Urteaga, DTN Routing Optimised by Human Routines: The HURRy Protocol in 13th International Conference, WWIC 2015, Malaga, Spain, May 25-27, 2015

[16] IEEE Standard 802.11u, "IEEE Standard for Information Technology-Telecommunications and information exchange between systems-Local and Metropolitan networks-specific requirements-Part II: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Amendment 9: Interworking with External Networks," 2011

[17] D. Camps-Mur et al., "Enabling always on service discovery: Wifi neighbor awareness networking," in IEEE Wireless Communications, , April 2015

[18] Arjuna Sathiaseelan, Liang Wang, Andrius Aucinas, Gareth Tyson, and Jon Crowcroft.. “SCANDEX: Service Centric Networking for Challenged Decentralised Networks”, In Proceedings of the 2015 Workshop on Do-it-yourself Networking: an Interdisciplinary Approach (DIYNetworking ’15), 2015

[19] Solomon Hykes and others. What is Docker? https://www. docker.com/whatisdocker/.

[20] NDN Packet Format Specification 0.2-alpha-3 documentation http://named-data.net/doc/ndn-tlv/data.html

[21] NDN HttpProxy. https://github.com/nameddata/ndnx/tree/master/apps/HttpProxy

[22] ccnd 0.7.2 forwarding strategy http://redmine.named-data.net/projects/nfd/wiki/CcndStrategy

[23] Information-Centric Networking (icnrg). https://datatracker.ietf.org/rg/icnrg/charter/

[24] Gareth Tyson, Nishanth Sastry, Ivica Rimac, Ruben Cuevas, and Andreas Mauthe. 2012. A survey of mobility in information-centric networks: challenges and research directions. In Proceedings of the 1st ACM workshop on Emerging Name-Oriented Mobile Networking Design - Architecture, Algorithms, and Applications (NoM '12). ACM, New York, NY, USA, 1-6. DOI=http://dx.doi.org/10.1145/2248361.2248363

[25] Y. T. Yu, J. Joy, R. Fan, Y. Lu, M. Gerla and M. Y. Sanadidi, "DT-ICAN: A Disruption-Tolerant Information-centric Ad-Hoc Network," 2014 IEEE Military Communications Conference, Baltimore, MD, 2014, pp. 1021-1026.
doi: 10.1109/MILCOM.2014.174

[26] M. Amadeo, C. Campolo, A. Iera and A. Molinaro, "Named data networking for IoT: An architectural perspective," Networks and Communications (EuCNC), 2014 European Conference on, Bologna, 2014, pp. 1-5.
doi: 10.1109/EuCNC.2014.6882665

[27] Fog Computing and the Internet of Things: Extend the Cloud to Where the Things Are. Cisco White Paper. Available at: https://www.cisco.com/c/dam/en_us/solutions/trends/iot/docs/computing-overview.pdf